Checklist for First-Time Bonneville Participants Riding Stock Motorcycles

Rider

__ leather boots at least 8” high

__ leather gloves with 3” gauntlet (must have fingers, no perforations, no textiles)

__ leather suit at least 8mm thick, with a full-circumference zipper; perforated leather OK

__ full-face helmet with face shield meeting Snell M2000 or M2005, ECE 22-05, or BSI Type A

__ shatterproof corrective eyewear if worn

Motorcycle (Run Whatcha Brung: two runs, not eligible for records)

__ ½” ball end on control levers (stock levers almost always are ball-ended)

__ handlebars must place hands outside of forks (stock motorcycles are all like this)

__ operator must have footpegs (stock motorcycles are all like this)

__ frame must be free of defects

__ frame stand and kickstand must be zip-tied or safety-wired before start

__ pump gasoline only (provided by the official supplier at Bonneville)

__ headlights removed

__ headlight covers taped over

__ positive ignition kill switch operable from the riding position (stock kill switch meets this)

__ mirrors removed (integrated mirrors must be taped)

__ OEM-equivalent tires required (must meet speed rating)

__ metal valve caps required

__ wheel discs or spinners prohibited

Motorcycle (Production Class: unlimited runs, record eligibility)

__ all previous requirements, plus:

__ stock displacement

__ exhaust pointed away from surface (stock exhaust meets this)

__ fuel shutoff operable from rider’s position (stock fuel switch meets this)

__ lanyard ignition shutoff required

__ fuel lines must be covered in fire sleeve

__ handlebars must have 10” between thumbs, extend 6” from forks, no more than 15” in front of or 4” lower than stock, and not allow rider to touch fairing or tank (stock handlebars should meet this)

__ racing number displayed, minimum 3” tall

__ seat height must be stock

__ transmission and engine drain plugs safety-wired

__ motorcycles in a class faster than 125 mph must have a steering damper

__ throttle must be self-closing (stock throttles meet this)

__ metal valve stems required

__ front and rear lighting intact but cross-taped, except non-integrated lighting which can be removed

__ fenders intact

__ stock wheels intact (but must meet speed rating)

__ stock brakes intact

__ stock air intake box intact

__ unmodified stock exhaust system intact

__ license plate, frame, and bracket, air cleaner element, and toolbox removed (optional)

__ chain guard required (stock guard meets this)

__ passener pegs removed

__ stock instruments intact

__ stock bodywork intact

__ stock engine components intact: cylinders, cases, heads, carburetor/throttle body, and starter